4Cam-Dev Rev A Errata

- Rotate Camera so that pin 1 is bottom right

- SRAM pin H6 is a no-connect – need to change OrCad library package

- P1 remove GPIO pins from I2C section

- P1 JTAG Vref thin trace – add resistor?

- P4 move Vin and GND next to each other

- Schematic power page, all thick traces, including to P4

- Change SRAM and Camera connections to CPLD for easy routing

- Add mounting holes #4 size

- Add logos and title, date, designer, owner, copyright, text to silkscreen

- fiducials ? Dan added fiducials and mounting holes on schematic

- parts sizes – 0604, LEDs 0805

- too many Vin pins, input caps may not be near actual Vin

- Move MCSn in P1 to SPI section

- LEDs on the bottom of the board, or camera on the bottom

- thermal reliefs on all pads on filled areas

Silkscreen notes:

Rotate logo and flip horizontal text to make camera right side up?

P1 silkscreen - "Slave SPI" remove the word "Slave" so

it just says "SPI"

Put a dividing line to the right of MCSn just like on the

left of it

1) LED labels - general purpose signals so use 
"LED1 LED2 LED3 LED4" 

2) P3 label - change "Video" to "VGA" 

3) P1 label - change pin 23 "GPIO1" to "C8" 

4) P1 label - change pin 22 "GPIO2" to "C10" 

5) P1 label - change pin 1 "3.3V" to "VREF" 

6) P1 silkscreen looks great! It will really make it 
easy for people to use. but some of the dividing 
lines are wrong compared to the schematic symbol 
i.e. move the PWR section to the right one pin and 
add a divider line for MCSn 

7) company logo attached, is jpg ok? 

8) product name "4Cam-Dev" 

9) white area rev number "Rev. A" 

10) Sigs? copyright? 
" 
Dan Meeks 9.2011 
© Craig Sullender 2011 
" 

11) P2 labels - label the expansion header signals like this 
(but check for changes if you back-annotated moved wires): 

Expansion Header pin - silkscreen label taken from CPLD pin number 
1 - T2 
2 - R3 
3 - T3 
4 - P4 
5 - GND 
6 - R4 
7 - T4 
8 - P5 
9 - T5 
10 - GND 
11 - M6 
12 - N6 
13 - R6 
14 - L7 
15 - GND 
16 - M7 
17 - N7 
18 - P7 
19 - R7 
20 - GND 
21 - T7 
22 - L8 
23 - M8 
24 - N8